INTERNET OUTAGE DR PROCEDURES V1.1

CONTENTS

1	INTERNET TROUBLESHOOTING	1

1.1	If network equipment has errors or not functioning	1

1.1.1	If WAN is reachable	2

1.1.2	If there is no outage	2

1.1.3	If Comcast Router is reachable.	2

2	INTERNET FAILOVER PROCEDURES	2

3	BOTH PRIMARY AND SECONDARY INTERNET DOWN	3

# INTERNET TROUBLESHOOTING

Follow these guidelines to trace problem:

- Ping local gateway to verify local router is online and reachable (both Internal and Public IP), If not reachable:

- Ping another server or device on the network to determine if there is a switch or DNS / Server problem, If not reachable:

- Do a physical inspection of the IDF and or MDF to verify Network equipment is on and functioning.

## If network equipment has errors or not functioning

- Open NOC Ticket

- Report to Network Admin and IT Manager

- *Network Admin may need to reboot equipment to bring back online.

- Verify if internet is back up by accessing websites.

- Attempt to ping a website by name and IP Address

- Check UDM Pro to verify WAN is showing connected

### If WAN is reachable

- Ping Comcast Router, If not reachable:

- Check Comcast outage map using this link https://www.xfinity.com/support/status-map.

### If there is no outage

- Open NOC Ticket

- Report to Network Admin and IT Manager

- *Network Admin may need to reboot equipment to bring back online.

- Verify if internet is back up by accessing websites.

- Attempt to ping a website by name and IP Address

- Check UDM Pro to verify WAN is showing connected

- Contact Comcast if there is still no internet.

- If outage go to section: “Internet Failover Procedures”

### If Comcast Router is reachable.

  - Ping Website by name

  - Ping website by IP

Note: If only reachable by IP may be a DNS problem.

# INTERNET FAILOVER PROCEDURES

This section lists steps to failover to AT&T line

- Verify internet is active on WAN 2 port on Barracuda and AT&T is pingable.

- Verify websites are pingable and accessible.

- Call into IVR to verify Comcast Phone service has failed over to backup line routed through VOIP.MS.

- If line has not failed over unplugging SIP ethernet from Comcast will work.

- Login to PBX.

- Under Settings and Network click verify Default Gateway to show AT&T IP.

- Verify that billing portal, paycor and quickbase are all functioning.

- If not open SOC ticket to have them verify if AT&T IP has been whitelisted on any sites not accessible.

- Reboot PBX if necessary.

- Update OpenVPN with AT&T IP so that end users can connect to VPN.

- Remote into ZSites and enable Site to Site VPN for AT&T Failover connection.

- When Internet is backup on Comcast

- First, Inform Users before continuing.

- Unplug AT&T line to fall back over to Comcast

- Verify internet is up by pinging and accessing websites.

- it may be necessary to reboot PBX again if phone issues occur.

(Sometimes requires more than one reboot)

- Change OpenVPN IP back to Comcast.

- Login back into Zsites and change Site to Site VPN.

# BOTH PRIMARY AND SECONDARY INTERNET DOWN

Follow this flow when both Comcast and AT&T lines are down.

  - Call into Comcast Business (855-211-1798)

  - Use Account# 934487785

  - SIP Trunking (for service needed) and ask for Technical Support

  - Change our Unreachable Destination number to 346-954-4211 to enable end user message that our internet is down. (Comcast Business Account Number at end of document.

  - Test by calling into IVR.

  - When internet is back up call back into Comcast and change number back to 844-709-7368 only used on PSP site.